Visual-Inertial Odometry (VIO)

Complete Code Walkthrough

Every line explained — what it does and why

# How to Read This Document

Each section follows this structure:

- The complete MATLAB code block exactly as it appears in the example

- A WHAT box — what the code is doing

- A WHY box — the mathematical or engineering reason it must be done this way

- Line-by-line or block-by-block commentary for every non-trivial statement

The document follows the three phases of the algorithm in order:

- Phase 1 — Structure from Motion (SfM) initialization

- Phase 2 — Camera-IMU alignment

- Phase 3 — Full Visual-Inertial optimization

# Section 1 — Setup and Initialization

## 1.1  Data loading and random seed

data = helperDownloadVIOData(); rng(0)

WHAT helperDownloadVIOData() fetches the Blackbird NYC Subway dataset (628 MB). It returns a struct containing: data.images (cell array of raw frames), data.gyroReadings, data.accelReadings, data.timeStamps, data.intrinsics (camera calibration), data.camToIMUTransform, and data.gTruth (ground truth poses). rng(0) fixes the random number generator seed.

WHY rng(0) is required because RANSAC is randomized — it samples point subsets randomly. Without a fixed seed, two runs of the same code give slightly different outlier rejection results, making the example non-reproducible.

## 1.2  Algorithm parameters

params = helperVIOParameters; status = struct( 'isFirstFrame', true, ... 'isMapInitialized', false, ... 'isIMUAligned', false, ... 'isMedianDepthEstimated', false); initialSceneMedianDepth = 4; viewID = 0; removedFrameIDs = []; allCameraTracks = cell(1,5000);

WHAT params is a struct of tuning constants: KLT block size, KLT pyramid levels, max bidirectional error, RANSAC iterations/threshold/confidence, optimization frequency, etc. status tracks which phase the algorithm is currently in. All flags start false/true as shown. initialSceneMedianDepth = 4 is the assumed real-world depth (in meters) of the scene when the SfM map is first created. This is a prior. viewID is a monotonically incrementing integer — every frame processed gets a unique ID regardless of whether it becomes a keyframe. allCameraTracks stores 2D point observations per view for later factor graph use.

WHY The status struct functions as an explicit state machine. Making the phase transitions visible (rather than implicitly tracking frame counts) makes the algorithm easier to debug — you can inspect status at any breakpoint and immediately know which phase is running. initialSceneMedianDepth = 4 is needed because monocular vision is scale-ambiguous. The SfM map will have an arbitrary internal scale. Before IMU alignment is complete, the system rescales the map so its median depth matches this prior (4 m). This prevents the factor graph from operating at a wildly wrong scale during the alignment phase.

## 1.3  Factor graph pre-allocation

fg = factorGraph; maxFrames = 10000; maxLandmarks = 100000; ids = helperGenerateNodeID(fg, maxFrames, maxLandmarks);

WHAT fg is a MATLAB factorGraph object — the back-end optimizer. helperGenerateNodeID pre-allocates four arrays of node IDs: ids.pose (one per frame), ids.vel (one per frame), ids.bias (one per frame), ids.point3 (one per 3D landmark). These are just integer handles — no actual data lives in them yet.

WHY Pre-allocating all node IDs at startup avoids the cost of dynamically registering nodes mid-loop. Since MATLAB can process up to 10,000 frames with up to 100,000 landmarks, bounding the arrays upfront lets the optimizer reserve memory efficiently. The factor graph has three node types per timestep — pose, velocity, and bias — because the IMU state is described by all three quantities. A pose node alone cannot encode IMU drift; the bias node captures how much the gyroscope and accelerometer are lying at any given moment.

## 1.4  KLT tracker and feature point manager

tracker = vision.PointTracker( MaxBidirectionalError = params.KLT_BiErr, NumPyramidLevels = params.KLT_Levels, BlockSize = params.KLT_Block); fpManager = helperFeaturePointManager( data.intrinsics, params, maxFrames, maxLandmarks); fpManager.DetectorFunc = @(I) helperDetectKeyPoints(I);

WHAT vision.PointTracker is MATLAB's Kanade-Lucas-Tomasi (KLT) optical flow tracker. It tracks a set of 2D image points from one frame to the next by minimizing pixel intensity difference in a local patch. MaxBidirectionalError: after tracking point P forward (frame t → t+1), the tracker also tracks the result backward (t+1 → t) and measures the round-trip displacement. If > threshold, the track is rejected. NumPyramidLevels: builds a Gaussian image pyramid so large inter-frame motions (which look like small motions at coarser scales) can still be tracked. BlockSize: the pixel patch size used for intensity comparison. fpManager is a custom class managing the sliding window of recent frames. It maintains 2D point tracks, assigns landmark IDs, and handles the bookkeeping of which points are visible in which frames.

WHY KLT is chosen over descriptor matching (SIFT, ORB) because it is significantly faster and works well for the small inter-frame motions typical in video. The cost is that tracks are sequential — you cannot match a frame to one from 100 frames ago without intermediate tracking. Bidirectional error checking is a cheap but effective outlier filter. A correctly tracked point should be near-perfectly round-trippable. Points on reflective surfaces, moving objects, or near occlusion boundaries often fail this check and should be dropped before they corrupt pose estimates.

## 1.5  Frame index setup

viewSet = imageviewset; startFrameIdx = 40; endFrameIdx = 1000; allFrameIDs = startFrameIdx:endFrameIdx;

WHAT imageviewset is a MATLAB container that stores camera poses indexed by view ID. It behaves like a map from viewID → rigidtform3d (rotation + translation). startFrameIdx = 40 means processing starts at frame 40 of the dataset, not frame 1.

WHY The Blackbird dataset only has synchronized IMU data starting from frame 40. Using earlier frames would mean camera-only processing without corresponding IMU data — the alignment step would then have insufficient overlap to succeed. endFrameIdx = 1000 is an example time limit. The full dataset has many more frames, but processing 960 frames is enough to demonstrate the complete algorithm while keeping example runtime reasonable.

PHASE 1 — Structure from Motion (SfM) Initialization

# Section 2 — SfM: Map Initialization Loop

Goal of this phase: establish the first relative pose between two keyframes so 3D points can be triangulated. This gives the factor graph its initial structure before IMU data is incorporated.

## 2.1  Outer loop condition

currFrameIdx = startFrameIdx; while ~status.isMapInitialized && currFrameIdx < endFrameIdx I = data.images{currFrameIdx}; viewID = viewID + 1; I = helperProcessImage(I, params, data.intrinsics);

WHAT This while-loop runs until either (a) the map is successfully initialized, or (b) we exhaust all frames. Inside the loop, each frame is loaded and preprocessed before any tracking or geometry is computed. viewID is incremented first — every frame, including non-keyframes, gets a unique ID. This ID is how the factor graph and view set cross-reference data.

WHY The loop condition ~status.isMapInitialized means the system keeps trying until it achieves sufficient parallax. It is possible for the first 5-10 frames to lack enough baseline (e.g., the camera barely moved) — the loop simply waits until a frame arrives with enough angular and translational separation from the first keyframe.

## 2.2  helperProcessImage — preprocessing every frame

I = helperProcessImage(I, params, data.intrinsics); % Inside helperProcessImage: % I = histeq(I); -- histogram equalization % I = undistortImage(I, intrinsics); -- lens distortion correction

WHAT Step 1 — histeq: redistributes pixel intensity values so the histogram is approximately uniform. Dark regions become brighter; over-exposed regions lose contrast. The result is an image where features are detectable across the full dynamic range. Step 2 — undistortImage: applies the inverse of the lens distortion model (radial coefficients k1, k2, k3 and tangential p1, p2) to every pixel, producing a geometrically correct image as if captured by an ideal pinhole camera.

WHY Histogram equalization: the Blackbird dataset includes a subway environment with inconsistent lighting — fluorescent tubes, dark tunnels, reflective floors. KLT tracking works by comparing patch intensities between frames. If a dark patch in frame t becomes a saturated patch in frame t+1 (due to a light source entering the frame), the tracker loses it. Equalization reduces this effect by normalizing intensities. Undistortion is mathematically mandatory. The fundamental matrix computation, essential matrix decomposition, and triangulation all rely on the pinhole camera model: a pixel at (u,v) corresponds to a ray in 3D along the direction K_inv * [u,v,1]. If the pixel is distorted, this ray is wrong, and all downstream geometry is corrupted. The error compounds multiplicatively over many frames.

## 2.3  First frame branch

if status.isFirstFrame currPoints = createNewFeaturePoints(fpManager, I); updateSlidingWindow(fpManager, I, currPoints, true(size(currPoints,1),1), viewID); initialize(tracker, currPoints, I); firstI = I; viewSet = addView(viewSet, viewID, rigidtform3d); status.isFirstFrame = false;

WHAT createNewFeaturePoints: runs the keypoint detector (FAST corners by default) on the image and returns Nx2 pixel coordinates. updateSlidingWindow: registers this frame and its keypoints in fpManager. The second argument true(...) marks all detected points as valid (since there are no tracks to reject yet). initialize(tracker, ...): seeds the KLT tracker with the detected points. From this call forward, tracker(I) will attempt to find where each of these points moved in the new image I. addView(viewSet, viewID, rigidtform3d): stores this frame in the view set with an identity transform — this frame IS the origin; all future poses are relative to it. status.isFirstFrame = false: ensures this block runs only once.

WHY The first frame is the coordinate origin by definition. There is no reference before it, so its pose must be identity (zero translation, zero rotation). Every subsequent pose will be expressed as 'camera moved X meters and rotated Y degrees relative to this first frame.' Seeding the KLT tracker at the first frame is necessary because the tracker is stateful — it needs to know which points to track and from which image. The initialize() call establishes this state.

## 2.4  Subsequent frames — KLT tracking

else [currPoints, validIdx] = tracker(I);

WHAT tracker(I) is the KLT forward pass. It takes the new image I and returns: currPoints — Nx2 array of where each previously-initialized point has moved to, validIdx — Nx1 boolean indicating which tracks are still valid (not lost, not exceeded bidirectional error threshold).

WHY KLT optical flow works by solving: for each point P in frame t, find displacement d such that the patch around P in frame t matches the patch around P+d in frame t+1. It solves this as a local least-squares problem. Points that cannot be matched within the error tolerance (e.g., they left the frame, went behind an occlusion, or their appearance changed dramatically) set validIdx=false.

## 2.5  Sliding window update

[removedFrameID, windowState] = updateSlidingWindow( fpManager, I, currPoints, validIdx, viewID); if (removedFrameID > fpManager.slidingWindowViewIDs(1)) removedFrameIDs(end+1) = removedFrameID; end

WHAT updateSlidingWindow inserts the current frame into the sliding window inside fpManager. If the window is already at maximum capacity, the oldest frame is evicted and returned as removedFrameID. It also runs fundamental matrix-based outlier rejection on the accumulated 2D tracks internally. windowState is a struct containing: isFirstFewViews (too few frames to estimate parallax yet), isEnoughParallax (sufficient baseline detected), isWindowFull (window at max capacity). The if-condition checks whether the removed frame was a non-keyframe and records it for visualization purposes.

WHY A sliding window is used instead of storing all frames because the computational cost of graph optimization scales with the number of nodes. Keeping only the N most recent frames bounds the optimization cost regardless of how long the algorithm runs. Frames that leave the window are 'marginalized out' — their constraints are absorbed into prior factors on the remaining nodes. Internally, fpManager also runs fundamental matrix-based outlier rejection here. This is a secondary filter on top of KLT's bidirectional error check. It exploits the global geometric constraint — if you have many point tracks, they must all be consistent with the same fundamental matrix. Tracks that violate it are likely due to dynamic objects (people moving) or repeated textures.

## 2.6  First-few-views branch

if windowState.isFirstFewViews viewSet = addView(viewSet, viewID, rigidtform3d);

WHAT While there are fewer frames than the minimum needed to assess parallax, each frame is accepted into the view set with an identity pose. No pose estimation is attempted.

WHY Parallax (angular separation) cannot be computed from a single frame or from two frames taken in nearly the same position. Attempting to estimate a relative pose when the baseline is near-zero produces a degenerate configuration — the essential matrix cannot be decomposed reliably because the epipole is at infinity. The system simply waits for the camera to move enough.

## 2.7  Enough parallax branch — RANSAC

elseif windowState.isEnoughParallax swIDs = getSlidingWindowIDs(fpManager); [matches1, matches2] = get2DCorrespondensesBetweenViews( fpManager, swIDs(end-1), swIDs(end)); inliersIndex = false(size(matches1,1),1); for k = 1:10 [f, inliers] = estimateFundamentalMatrix( matches1, matches2, Method = 'RANSAC', NumTrials = params.F_Iterations, DistanceThreshold = params.F_Threshold, Confidence = params.F_Confidence); if length(find(inliersIndex)) < length(find(inliers)) inliersIndex = inliers; fundamentalMatrix = f; end end

WHAT get2DCorrespondensesBetweenViews retrieves all 2D point matches between the second-to-last and last frames in the sliding window. Each row is [u1,v1] in frame A and the corresponding [u2,v2] in frame B. estimateFundamentalMatrix with Method=RANSAC implements the standard 8-point algorithm inside a RANSAC loop. It returns the best 3x3 fundamental matrix F and a boolean vector marking which point pairs are inliers. The outer for k=1:10 loop runs RANSAC 10 separate times with independent random seeds. At each iteration, if the new run found more inliers than the best so far, the result is updated. The final inliersIndex is the best inlier set across all 10 runs.

WHY Why RANSAC? Because some of the tracked point pairs are wrong — a moving person, a reflection, or a repeated tile pattern. RANSAC handles outliers by hypothesis: pick a minimal set of points (8 for the fundamental matrix), fit F, count how many other points agree. Repeat many times and take the best. The name comes from Random Sample Consensus. Why run RANSAC 10 times? RANSAC is probabilistic. Each run has some chance of drawing a minimal sample that includes an outlier, producing a suboptimal F. Running 10 independent times and keeping the best is a simple ensemble technique that pushes the probability of finding the true solution much higher. With 30% outliers, one RANSAC run at 500 iterations has ~99% success probability; 10 runs raises this to ~99.999%. The fundamental matrix F encodes the epipolar constraint: for any correctly matched pair (x1, x2), x2^T * F * x1 = 0. Points violating this by more than params.F_Threshold pixels are labeled outliers.

## 2.8  Relative pose estimation from F

inlierPrePoints = matches1(inliersIndex, :); inlierCurrPoints = matches2(inliersIndex, :); relPose = estrelpose( fundamentalMatrix, data.intrinsics, inlierPrePoints, inlierCurrPoints); viewSet = addView(viewSet, viewID, relPose); status.isMapInitialized = true; break

WHAT inlierPrePoints and inlierCurrPoints are the filtered subsets — only the point pairs that RANSAC deemed geometrically consistent. estrelpose: (1) computes the Essential matrix E = K^T * F * K, (2) decomposes E via SVD into four candidate (R,t) pairs, (3) tests all four using the positive-depth cheirality test (points must be in front of both cameras), and (4) returns the unique valid relative pose as a rigidtform3d. addView stores this pose in the view set. status.isMapInitialized = true and break exit both the if-block and the while-loop.

WHY The fundamental matrix F maps between pixel coordinates. The essential matrix E operates in normalized camera coordinates (pixel coords divided by the camera intrinsic matrix K). E has fewer degrees of freedom than F — it only encodes the physical 3D rigid body motion (rotation + translation direction), not the camera parameters. Decomposing E gives the actual relative pose. The translation t is recovered only up to scale — this is the fundamental monocular ambiguity. If the camera moved 10 cm or 10 m, the image looks identical. The scale will be resolved in Phase 2 when IMU data is incorporated. The cheirality test eliminates 3 of the 4 (R,t) candidates. E decomposition gives two possible rotations and two possible translation directions (sign ambiguity). Only one combination places the triangulated points in front of both cameras — the others produce points behind one or both cameras, which is physically impossible.

NOTE RANSAC in this algorithm runs ONLY HERE — once, to initialize the map. After this point the system switches to PnP (3D-2D geometry) which does not require RANSAC because it has existing 3D landmarks to constrain the solution more strongly.

PHASE 2 — Camera-IMU Alignment

# Section 3 — Camera-IMU Alignment Loop

Goal: take the scale-ambiguous SfM poses and transform them into the IMU navigation frame with correct metric scale. Estimate initial gyroscope and accelerometer biases.

## 3.1  IMU parameter setup

cameraInformation = ((data.intrinsics.K(1,1)/1.5)^2) * eye(2); imuParams = factorIMUParameters( SampleRate = 100, GyroscopeNoise = 0.1, GyroscopeBiasNoise = 3e-3, AccelerometerNoise = 0.3, AccelerometerBiasNoise= 1e-3, ReferenceFrame = 'ENU'); readyToAlignCameraAndIMU = false;

WHAT cameraInformation is the 2x2 information matrix (inverse covariance) for camera projection factors. It tells the optimizer how much to trust a 2D observation. Larger diagonal = higher trust. Scaled by focal length squared because pixel noise in focal-length-normalized coordinates is ~1/f times smaller than in pixel units. factorIMUParameters defines the stochastic model for the IMU: noise densities for gyro and accelerometer (continuous-time white noise on the sensor readings), and noise densities for their biases (random walk — biases drift slowly). SampleRate=100 means 100 Hz IMU. ReferenceFrame=ENU sets the world frame as East-North-Up.

WHY These noise parameters encode the relative trust between camera and IMU in the joint factor graph. If gyro noise is set too low, the optimizer over-trusts the IMU and camera measurements are overridden. If set too high, the IMU provides little correction. Real sensor datasheets provide these figures; the values here are tuned for the Blackbird dataset's specific IMU hardware. ENU (East-North-Up) is the standard local navigation frame. Gravity points in the -Z direction. This convention ensures that the gravity vector, when estimated from IMU data, aligns with the expected direction.

## 3.2  Triangulation of new 3D points

while ~status.isIMUAligned [newXYZ, newXYZID, newPointViews, newPointObs] = ... triangulateNew3DPoints(fpManager, viewSet);

WHAT triangulateNew3DPoints examines all 2D point tracks in fpManager that are visible in at least 2 keyframes with known poses but do not yet have an associated 3D point. For each such track, it uses the known camera poses (from viewSet) and the 2D observations to compute the 3D position via direct linear transform (DLT) triangulation. Returns: newXYZ (Nx3 world coordinates), newXYZID (IDs for the factor graph), newPointViews (which frames see each new point), newPointObs (the 2D pixel observations in each frame).

WHY Triangulation: given a 3D point P and two cameras with projection matrices P1 and P2, the image points x1 = P1*P and x2 = P2*P. Given x1 and x2, solve for P. This is done by forming a linear system [x1 cross P1; x2 cross P2] * P = 0 and solving via SVD. This must run continuously as new frames arrive because: (1) each new frame adds new 2D tracks, and (2) some tracks that were only visible in one frame before become visible in a second frame now, enabling triangulation for the first time.

## 3.3  Adding camera factors to the graph

if ~isempty(newXYZ) for pIte = 1:length(newPointViews) allCameraTracks{newPointViews(pIte)} = ... [allCameraTracks{newPointViews(pIte)}; newPointObs{pIte}]; end obs = vertcat(newPointObs{:}); fCam = factorCameraSE3AndPointXYZ( [ids.pose(obs(:,1)) ids.point3(obs(:,2))], data.intrinsics.K, Measurement = obs(:,3:4), Information = cameraInformation); addFactor(fg, fCam); end

WHAT factorCameraSE3AndPointXYZ creates one factor per 2D observation. Each factor connects a pose node (the camera position at frame t) and a 3D point node (the landmark in world space). Its residual is: predicted_pixel - observed_pixel, where predicted_pixel = project(K, pose, point3D). The factor is added to the graph with addFactor. At this point no optimization has run — the factor is only registered as a constraint.

WHY Camera projection factors are the visual measurements in the factor graph. Each one says: 'if the camera was at pose P and the 3D point is at position X, then I should observe the point at pixel location u.' The optimizer adjusts both P and X until all these constraints are satisfied as well as possible. Using the information matrix (rather than a covariance matrix) avoids matrix inversion inside the optimizer's inner loop. Information = inverse covariance. Higher information = smaller covariance = more reliable measurement.

## 3.4  PnP pose estimation (pre-IMU alignment)

% In the else branch (IMU not yet aligned): x3D = getXYZPoints(fpManager, pointIds(isTriangulated)); c2D = currPoints(isTriangulated, :); [currPose, ii] = helperEstimateCameraPose( params, c2D, x3D, data.intrinsics, currPose, ii); viewSet = addView(viewSet, viewID, currPose);

WHAT getXYZPoints retrieves the 3D world coordinates for all keypoints in the current frame that have already been triangulated (isTriangulated = true). helperEstimateCameraPose runs Perspective-n-Point (PnP): given N pairs of (3D world point, 2D image observation), find the camera pose (R, t) that minimizes reprojection error. MATLAB's estworldpose implements this with iterative optimization after an algebraic initialization.

WHY After map initialization, you no longer need RANSAC + fundamental matrix to get poses. You already have 3D landmarks in the world. PnP is a direct solver: given I know the 3D structure, and I can see these points at these pixel locations, what camera pose produced this image? This is geometrically much better constrained than the 2D-2D problem. PnP uses 3D-2D correspondences (known 3D point → observed 2D pixel) instead of 2D-2D correspondences. This requires 3 point pairs minimum (EPnP uses 4) vs. 8 for the fundamental matrix. Fewer required points → less sensitive to outliers.

## 3.5  Sliding window bundle adjustment (camera only)

if helperDecideToRunGraphOptimization( currFrameIdx, newPointsTriangulated, params) nodeIdsToOptimize = ids.pose(slidingWindowViewIDs); xyzIds = getPointIdsInViews(fpManager, slidingWindowViewIDs); fg.nodeState(ids.point3(xyzIds), xyz); helperFixNodes(fg, ids.pose, slidingWindowViewIDs, windowState.isWindowFull); optiInfo = optimize(fg, nodeIdsToOptimize, params.SolverOpts); helperUnFixNodes(fg, ids.pose, slidingWindowViewIDs, windowState.isWindowFull); end

WHAT helperDecideToRunGraphOptimization returns true if: (a) we are within the first 250 frames (optimize every frame), OR (b) currFrameIdx is a multiple of params.optimizationFrequency, OR (c) new 3D points were just triangulated. nodeIdsToOptimize limits optimization to only the pose nodes in the current sliding window — not all frames ever processed. helperFixNodes pins the oldest 11 pose nodes in the window (or just the first, if the window is not yet full). Fixed nodes are not adjusted during optimization. optimize(fg, nodeIdsToOptimize, ...) runs Levenberg-Marquardt (or Gauss-Newton) to minimize the sum of squared residuals across all factors connected to the selected nodes.

WHY Fixing the oldest nodes is essential to prevent gauge freedom. A factor graph with only relative constraints (camera sees point, point constrains pose) has an infinite number of equivalent solutions — you can translate or rotate the entire solution and get the same factor values. Fixing one node anchors the coordinate system. Sliding window optimization (only the last N frames) trades accuracy for speed. A full batch optimization over all 1000 frames would give a marginally better solution but would grow quadratically slower with each new frame. For real-time operation, bounding the window size bounds the per-step compute time. Optimizing every frame for the first 250 frames (from helperDecideToRunGraphOptimization) ensures the initial SfM map is high quality before IMU alignment begins. Poor initial poses lead to failed alignment.

## 3.6  Median depth normalization

if ~status.isMedianDepthEstimated status.isMedianDepthEstimated = true; xyz = fg.nodeState(ids.point3(xyzIds)); EstimatedMedianDepth = median(vecnorm(xyz.')); [posesUpdated, xyz] = helperTransformToNavigationFrame( helperUpdateCameraPoseTable(...), xyz, rigidtform3d, initialSceneMedianDepth / EstimatedMedianDepth); end

WHAT vecnorm(xyz.') computes the Euclidean distance from the origin for each 3D point. median(...) gives the median depth of the scene in the SfM-internal coordinate system. helperTransformToNavigationFrame scales all poses and 3D points by the ratio initialSceneMedianDepth / EstimatedMedianDepth. If the SfM map says the median depth is 0.2 (arbitrary units) but the prior says it should be 4 meters, every point and pose is scaled by 4/0.2 = 20.

WHY This step runs exactly once — the first time optimization produces 3D points. It applies a rough metric scale to the map using the prior initialSceneMedianDepth = 4 m. Without this step, the SfM map might have an internal scale of 0.001 or 1000, neither of which is compatible with IMU measurements in meters per second squared. The factor graph cannot mix a map in arbitrary units with IMU data in SI units — the Levenberg-Marquardt optimizer would produce nonsensical results because the Jacobian scaling would be off by orders of magnitude. This is only a rough fix. The exact metric scale will be computed precisely in Section 3.8 using the IMU data. This step just ensures the map is in the right ballpark so the alignment algorithm converges.

## 3.7  Detection of alignment readiness

if windowState.isWindowFull && ~readyToAlignCameraAndIMU readyToAlignCameraAndIMU = true; helperPlotCameraPosesAndLandmarks(...); end

WHAT This block triggers once — when the sliding window first becomes full. It sets the flag readyToAlignCameraAndIMU = true. On the next iteration, the IMU alignment code will run.

WHY IMU alignment needs multiple frames with known camera velocities. Camera velocity is implicitly encoded in the sequence of poses — if you have too few poses, the velocity estimate from finite differencing is noisy. A full sliding window guarantees enough pose history to compute a reliable velocity estimate for the alignment equations.

## 3.8  Camera-IMU alignment: gravity and scale

[gyro, accel] = helperExtractIMUDataBetweenViews( data.gyroReadings, data.accelReadings, data.timeStamps, allFrameIDs(swIDs)); [gRot, scale, info] = estimateGravityRotationAndPoseScale( camPoses, gyro, accel, SensorTransform = data.camToIMUTransform, IMUParameters = imuParams); disp('Estimated scale: ' + scale);

WHAT helperExtractIMUDataBetweenViews queries the IMU data arrays for all gyro and accelerometer readings that fall between consecutive camera frame timestamps in the sliding window. estimateGravityRotationAndPoseScale is the core alignment function. It solves a linear system relating camera pose differences (velocities computed from pose sequence) to IMU pre-integrated measurements. The unknowns are: (1) gravity rotation R_g (how is the global gravity vector oriented relative to the first camera frame), (2) pose scale s (multiply all camera translations by s to get metric units), and (3) initial velocity v0 and bias b.

WHY The alignment problem in closed form: between two frames i and i+1 separated by time dt, the IMU preintegration gives delta_R, delta_v, delta_p. The camera gives a relative translation t_cam. The constraint is: s * t_cam = delta_p + delta_v * dt + 0.5 * (R_g * g) * dt^2. Stacking these constraints for all sliding window frame pairs gives a linear system in [s, R_g, v0, b] that can be solved by least squares. The scale factor of 1.8671 printed during execution means the SfM camera translations were about 1.87 times too small compared to IMU-derived distances. This is within a typical range for monocular systems where the initial depth prior was approximate.

## 3.9  Applying alignment and adding IMU factors

if info.IsSolutionUsable && scale > 1e-3 status.isIMUAligned = true; [posesUpdated, xyz] = helperTransformToNavigationFrame( posesUpdated, xyz, gRot, scale); viewSet = updateView(viewSet, posesUpdated); for k = 1:length(gyro) nId = [ids.pose(swIDs(k)), ids.vel(swIDs(k)), ids.bias(swIDs(k)), ids.pose(swIDs(k+1)),ids.vel(swIDs(k+1)), ids.bias(swIDs(k+1))]; fIMU = factorIMU(nId, gyro{k}, accel{k}, imuParams, SensorTransform=data.camToIMUTransform); fg.addFactor(fIMU); end end

WHAT info.IsSolutionUsable checks the condition number of the alignment linear system — a poorly conditioned system means the camera moved in a degenerate trajectory (e.g., pure rotation without translation) and the scale cannot be reliably estimated. scale > 1e-3 prevents accepting a degenerate near-zero scale which would collapse the entire map to a point. helperTransformToNavigationFrame applies gRot (gravity rotation) and scale to all poses and 3D points, converting everything to the IMU navigation frame in metric units. The for-loop creates one factorIMU object per pair of consecutive sliding window frames. Each factorIMU encodes the IMU preintegration — the combined effect of all gyro and accelerometer readings between those two timestamps.

WHY IMU preintegration (inside factorIMU): integrating raw accelerometer data directly is numerically unstable because noise accumulates. Instead, you preintegrate all IMU readings between two keyframe timestamps into a single delta_R (rotation change), delta_v (velocity change), delta_p (position change). This preintegrated measurement is stable and can be represented as a single factor in the graph. If the bias estimate changes during optimization, the preintegration can be corrected via first-order perturbation without reintegrating from scratch. SensorTransform=data.camToIMUTransform: the camera and IMU are not co-located on the drone. The IMU factor must account for the rigid body offset between the camera optical center and the IMU center of mass. This transform (typically a 6-DOF extrinsic calibration) converts between the two coordinate frames.

## 3.10  Initial IMU bias estimation via optimization

fixNode(fg, ids.pose(swIDs)); fixNode(fg, ids.point3(xyzIds)); fVelPrior = factorVelocity3Prior(ids.vel(swIDs(1))); fBiasPrior = factorIMUBiasPrior(ids.bias(swIDs(1))); addFactor(fg, fVelPrior); addFactor(fg, fBiasPrior); soll1 = optimize(fg, params.SolverOpts); fixNode(fg, ids.pose(swIDs), false); fixNode(fg, ids.point3(xyzIds), false); fixNode(fg, ids.pose(swIDs(1))); soll = optimize(fg, params.SolverOpts); fixNode(fg, ids.pose(swIDs(1)), false);

WHAT Round 1 (soll1): all pose and 3D point nodes are fixed. Only velocity and bias nodes are free to move. The optimizer finds the velocity and bias values that best explain the IMU measurements given the already-known camera trajectories. factorVelocity3Prior: a soft constraint anchoring the first velocity node near zero (the system is assumed to start from near-rest). factorIMUBiasPrior: a soft constraint anchoring the first bias near zero. Round 2 (soll): unfix poses and points, but fix only the very first pose (to prevent gauge freedom). Now everything is optimized jointly — poses, 3D points, velocities, biases all move together.

WHY The two-round optimization is a warm-start strategy. If you try to jointly optimize everything from a cold start (random biases, approximate poses), the optimizer may get stuck in a local minimum because the initial bias estimate is far from the truth and produces large IMU residuals that mislead the optimizer. By first solving for biases with fixed poses (well-conditioned, easier problem), then refining everything jointly, the optimizer starts the second round with a good bias estimate and converges quickly to the global solution. Velocity priors are needed because camera observations do not directly constrain absolute velocity — they only constrain pose differences. Without a prior, the first velocity node is unobserved and the system is rank-deficient.

PHASE 3 — Visual-Inertial Optimization (Main Loop)

# Section 4 — Visual-Inertial Optimization Loop

Goal: process all remaining frames using both camera and IMU data for tightly coupled optimization, producing the final trajectory estimate.

## 4.1  IMU factor creation per frame

while currFrameIdx < endFrameIdx + 1 swIDs = getSlidingWindowIDs(fpManager); svs = swIDs((end-1):end); [gyro, accel] = helperExtractIMUDataBetweenViews( data.gyroReadings, data.accelReadings, data.timeStamps, allFrameIDs(svs)); nodeID = [ids.pose(svs(1)) ids.vel(svs(1)) ids.bias(svs(1)) ids.pose(svs(2)) ids.vel(svs(2)) ids.bias(svs(2))]; fIMU = factorIMU(nodeID, gyro{1}, accel{1}, imuParams, SensorTransform=data.camToIMUTransform); fg.addFactor(fIMU);

WHAT svs = swIDs((end-1):end) extracts the last two frame IDs from the sliding window — the previous frame and the current frame. A factorIMU is created connecting 6 nodes: [pose_t, vel_t, bias_t, pose_{t+1}, vel_{t+1}, bias_{t+1}]. This factor's residual is the difference between: (a) what the IMU predicts happened between these two frames, and (b) what the node states say happened.

WHY Every new frame adds one IMU factor to the graph. This is the key difference from Phase 2 — there, IMU factors were added retroactively at alignment time. Now they are added in real-time as each frame is processed, keeping the IMU constraints current. The factorIMU connects 6 nodes instead of just 2 pose nodes (as a camera factor would) because IMU measurements constrain not just position but also velocity and bias. A pose-only connection would be unable to correctly model IMU dynamics.

## 4.2  IMU pose prediction

prevP = nodeState(fg, ids.pose(svs(1))); prevVel = nodeState(fg, ids.vel(svs(1))); prevBias = nodeState(fg, ids.bias(svs(1))); [pp, pv] = fIMU.predict(prevP, prevVel, prevBias);

WHAT nodeState retrieves the current estimates for the previous frame's pose, velocity, and bias from the factor graph. fIMU.predict integrates the gyroscope and accelerometer readings from time t to t+1 using the current bias estimates to produce: pp (predicted pose at t+1), pv (predicted velocity at t+1).

WHY The IMU prediction serves as an initial guess for the current frame's pose before any camera measurements are processed. This is valuable because: (a) at high speeds or aggressive maneuvers, the camera-only estimate (from PnP) may be far from the truth due to motion blur or feature loss, and (b) a good initial guess makes the PnP solver and the subsequent bundle adjustment converge faster and more reliably. The integration uses the current bias estimate (not true bias, which is unknown). As the optimizer refines the bias nodes over time, the predictions improve — this is the IMU bias correction loop.

## 4.3  PnP with IMU as initial guess

x3D = getXYZPoints(fpManager, pointIds(isTriangulated)); c2D = currPoints(isTriangulated, :); [currPose, velRefined, biasRefined, ii] = ... helperBundleAdjustmentMotion( x3D, c2D, data.intrinsics, size(I), pp, pv, prevP, prevVel, prevBias, fIMU); fg.nodeState(ids.vel(viewID), velRefined); fg.nodeState(ids.bias(viewID), biasRefined); viewSet = addView(viewSet, viewID, currPose);

WHAT helperBundleAdjustmentMotion is a motion-only bundle adjustment: it optimizes the current frame's pose, velocity, and bias while holding all 3D landmarks fixed. It uses both: (1) the camera reprojection error (2D observations vs. projected 3D points), and (2) the IMU residual from fIMU (predicted state vs. optimized state). pp (IMU predicted pose) is passed as the initial guess, so the solver starts close to the truth. velRefined and biasRefined are the updated velocity and bias estimates for this frame, stored back into the factor graph via nodeState.

WHY Motion-only bundle adjustment (fixing landmarks, optimizing only the current pose) is faster than full bundle adjustment (optimizing both). It is appropriate here because: (a) the sliding window handles the full optimization periodically, (b) a rough per-frame pose estimate is sufficient for the next tracking step, and (c) the IMU prediction already provides a near-correct starting point, so few iterations are needed. The joint camera+IMU optimization within this function is what makes the system 'tightly coupled.' A loosely coupled system would estimate the camera pose separately from the IMU and then fuse the two pose estimates. Tightly coupled fuses the raw measurements (pixels and IMU readings) in a single optimization, exploiting their complementary properties more fully.

## 4.4  Triangulation and factor addition

[newXYZ, newXYZID, newPointViews, newPointObs] = ... triangulateNew3DPoints(fpManager, viewSet); % ... add camera factors for new points (same as Phase 2) ... nodeState(fg, ids.pose(slidingWindowViewIDs(end)), ...);

WHAT Triangulation runs every frame (same as Phase 2) to continuously add new 3D landmarks as more views of existing 2D tracks accumulate. nodeState(...) updates the current frame's pose node in the factor graph with the estimate from helperBundleAdjustmentMotion. This provides the optimizer with a good initial guess for the next optimization step.

WHY Continuous triangulation maintains map density. As the camera moves, some landmarks leave the field of view (their tracks terminate) and new features enter. Without continuously adding new landmarks, the map becomes sparse and PnP has fewer 3D-2D correspondences, increasing pose estimation uncertainty.

## 4.5  Sliding window visual-inertial optimization

if helperDecideToRunGraphOptimization(...) nodeIdsToOptimize = ids.pose(slidingWindowViewIDs); xyzIds = getPointIdsInViews(fpManager, slidingWindowViewIDs); fg.nodeState(ids.point3(xyzIds), xyz); helperFixNodes(fg, ids.pose, slidingWindowViewIDs, windowState.isWindowFull); fixNode(fg, ids.vel(slidingWindowViewIDs(1))); fixNode(fg, ids.bias(slidingWindowViewIDs(1))); optiInfo = optimize(fg, nodeIdsToOptimize, params.SolverOpts); helperUnFixNodes(fg, ids.pose, slidingWindowViewIDs, windowState.isWindowFull); % update viewSet and fpManager with optimized values end

WHAT This is the full visual-inertial optimization step. It is the same sliding window optimization as Phase 2, with one addition: IMU factors are now also active in the graph. fixNode for vel and bias of the first sliding window frame: pins the velocity and bias of the oldest frame in the window. This ensures the IMU factors have a stable anchor — without it, the velocity/bias nodes at the boundary float freely and the optimizer may drift. After optimization, viewSet and fpManager are updated with the refined poses and 3D points.

WHY With both factorCameraSE3AndPointXYZ and factorIMU active in the same graph, the optimizer is simultaneously enforcing: (1) camera observations (every tracked 2D point should project onto its 3D landmark), and (2) IMU dynamics (the pose, velocity, and bias at time t+1 should be consistent with integrating the IMU readings from time t given the bias at time t). These two constraints are complementary: the camera corrects slow IMU drift; the IMU handles fast motion where the camera measurement is unreliable.

# Section 5 — Results and Accuracy Evaluation

## 5.1  Ground truth comparison

helperPlotAgainstGroundTruth(viewSet, data.gTruth, ... data.camToIMUTransform, allFrameIDs(viewSet.Views.ViewId), axf, removedFrameIDs); helperComputeErrorAgainstGroundTruth(...); % Output: % Absolute RMSE for key frame trajectory (m): 0.23326 % Percentage of median scale error: 3.0064

WHAT Absolute Trajectory Error (ATE): for each keyframe, compute the Euclidean distance between the estimated position and the ground truth position. RMSE across all keyframes = 0.233 m. Scale error: compute the median ratio of estimated trajectory segment lengths to ground truth segment lengths. 3.0% scale error means the estimated total path length is 3% longer or shorter than true.

WHY 0.233 m RMSE over a ~100m subway trajectory represents ~0.23% position error. This is excellent for a monocular system (no stereo, no GPS, no prior map). 3% scale error means the IMU alignment largely succeeded — a naive monocular camera system without IMU would have scale errors of 10-100% because scale is completely unobservable from images alone. These numbers validate the entire pipeline: the preprocessing gave good feature tracks, RANSAC successfully initialized the map, the alignment correctly recovered metric scale, and the visual-inertial optimization refined the trajectory without drifting.

# Section 6 — Key Helper Functions Explained

## helperDecideToRunGraphOptimization

function shouldOptimize = helperDecideToRunGraphOptimization( currFrameIdx, newPointsTriangulated, params) numberOfInitialFrames = 250; shouldOptimize = (currFrameIdx < numberOfInitialFrames) ¦¦ (mod(currFrameIdx, params.optimizationFrequency) == 0) ¦¦ newPointsTriangulated; end

WHAT Returns true under three conditions: (1) still in the first 250 frames, (2) current frame is a multiple of the optimization frequency, (3) new 3D points were just triangulated.

WHY Condition 1: the initial map must be high quality — optimize aggressively early. Condition 2: periodic optimization keeps estimates fresh without running every frame (too slow). Condition 3: new landmarks introduce new camera factors into the graph; optimizing immediately absorbs these constraints and updates all connected poses.

## helperFixNodes / helperUnFixNodes

function helperFixNodes(factorGraph, poseIds, slidingWindowIds, isWindowFull) if isWindowFull fixNode(factorGraph, poseIds(slidingWindowIds(1:11))); else fixNode(factorGraph, poseIds(slidingWindowIds(1))); end end

WHAT When the window is full: fix the oldest 11 pose nodes. When the window is still filling up: fix only the very first pose.

WHY Fixing 11 nodes when the window is full is a numerical stability choice. With only one fixed node and many IMU factors forming a chain, small residuals in the middle of the chain can cause the overall optimization to drift. Fixing more nodes at the boundary makes the problem better conditioned. The number 11 is an empirically tuned hyperparameter for this dataset.

## helperGenerateNodeID

function ids = helperGenerateNodeID(fg, maxFrames, maxLandmarks) ids.pose = generateNodeID(fg, [maxFrames 1]); ids.vel = generateNodeID(fg, [maxFrames 1]); ids.bias = generateNodeID(fg, [maxFrames 1]); ids.point3 = generateNodeID(fg, [maxLandmarks 1]); end

WHAT Pre-allocates all node ID arrays in one call. Each generateNodeID call reserves a contiguous block of integer IDs in the factor graph without setting any initial values.

WHY Using pre-allocated integer IDs (rather than dynamic insertion) allows O(1) node lookup during the main loop: ids.pose(viewID) immediately gives the factor graph node ID for that frame. Dynamic insertion would require a hash map lookup on every access.

# Section 7 — Algorithm Operations Summary

The following table summarizes every major operation, when it runs, and what it depends on.

| Operation | When it runs | Requires |
| --- | --- | --- |
| Histogram equalization | Every frame, always | Raw image |
| Lens undistortion | Every frame, always | Camera intrinsics |
| KLT tracking | Every frame after first | Initialized tracker + prev frame |
| Sliding window update | Every frame after first | KLT valid tracks |
| RANSAC + fund. matrix | ONCE — at first enough-parallax event | 2D-2D matches, >8 point pairs |
| Essential matrix decomp. | ONCE — immediately after RANSAC | Fundamental matrix + intrinsics |
| Triangulation | Phase 2+3, whenever new 2D-2D matches exist | ≥2 frames with known poses |
| Camera-only bundle adj. | Phase 2, periodic + on new landmarks | Camera factors in graph |
| Gravity + scale alignment | ONCE — when sliding window first full | Multiple frames of cam poses + IMU |
| Initial bias estimation | ONCE — immediately after alignment | IMU factors + aligned poses |
| PnP pose estimation | Phase 2+3, every frame | Triangulated 3D points + 2D obs |
| IMU prediction | Phase 3, every frame | Previous pose/vel/bias + IMU |
| Motion-only BA (VI) | Phase 3, every frame | IMU prediction + PnP + camera factors |
| Full VI optimization | Phase 3, periodic + on new landmarks | Both camera and IMU factors |